# 7.3 Final Lab Practical

Overview

This assignment is the culminating, summative lab assignment for IT 350 – Network Configuration and Design.

In this final lab, you will demonstrate your ability to integrate, configure, verify, and troubleshoot a complete network using the technologies and skills you have developed throughout the course. You are expected to approach this work the way a network technician or junior network engineer would in a real workplace environment.

Files

Download the two files provided for this assignment:

- IT 350 - Final Lab Practical.docxDownload IT 350 - Final Lab Practical.docx

- IT 350 - Final Lab Practical - Topology.pktDownload IT 350 - Final Lab Practical - Topology.pkt

Scenario

- Please see IT 350 - Final Lab Practical.docx for the scenario you will be working with.

Purpose of the Worksheet and Screenshots

- The Final Lab Practical worksheet defines the requirements for this assignment and provides examples of verification evidence, including screenshots, to help you confirm your configuration.

- Screenshots shown in the worksheet illustrate what correct results look like; you are not required to submit screenshots or the worksheet itself.

- Final submission consists of the Packet Tracer (.pkt) file and the show run output for all required devices, as described in the final step of the worksheet.

Instructions

- Download the two files provided in this assignment.

- Follow the instructions listed in the document, IT 350 - Final Lab Practical.docx, to complete your project (to satisfy all requirements and achieve full connectivity between all PCs in the network).

- Capture the required "show run" output file(s):

  - Save your show run on your routers and switch with a wr mem and copy them to a text file.

    - You may use one text file for all devices or may upload a separate text file for each device.

      - If you use one file, please clearly label the beginning of each device’s output with the device name so that it is clear where each device’s show run begins and ends.

      - If you use multiple files, please name them with the hostname of the device.

- Submit the following to Canvas

  - Your completed Packet Tracer (.pkt) file

  - The “show run” output file(s) for all devices (R1-SITE1, R2-SITE2, S1-SITE1, S2-SITE1, and S3-SITE1)

Grading

Your assignment will be evaluated according to the criteria outlined in the rubric.

Rubric

7.3 Final Lab Practical - Rubric

| 7.3 Final Lab Practical - Rubric |  |  |
| --- | --- | --- |
| Criteria | Ratings | Pts |
| This criterion is linked to a Learning OutcomeDevice Hostnames | 8 ptsFull MarksAll required routers and switches are configured with hostnames that exactly match the topology labels. The submitted evidence (for example, output from the command "show cdp neighbor" captured on S1-SITE1) clearly shows all expected device names with no discrepancies. 7 ptsMost MarksHostnames are correctly configured on most required devices, but one device has a minor hostname issue (such as a typo, formatting difference, or missing hostname). Evidence shows the remaining device hostnames match the topology labels. 5 ptsModerate MarksHostnames are configured on some required devices, but multiple devices have missing or incorrect hostnames. Evidence shows partial completion of the hostname requirement. 3 ptsSome MarksMinimal hostname configuration is present. One or more devices may show a hostname change, but most required devices do not match the topology labels or evidence is insufficient. 0 ptsNo MarksNo evidence is provided showing hostname configuration, or the submitted evidence does not demonstrate that any required device hostnames were configured. | 8 pts |
| This criterion is linked to a Learning OutcomeInterface Status (Green Links) | 8 ptsFull MarksAll required interfaces show an up/up state, and the submitted evidence (for example, a screenshot of the logical view) clearly shows green links on all devices with no interface issues present. 7 ptsMost MarksMost required interfaces show a green (up/up) status, but one interface is down or incorrectly configured. The submitted evidence otherwise demonstrates that the majority of interfaces are operational. 5 ptsModerate MarksSome interfaces show a green (up/up) status, but multiple required interfaces are down or not correctly configured. The submitted evidence shows partial completion of the interface status requirement. 3 ptsSome MarksMinimal progress toward correct interface status is evident. One or more interfaces may be up, but most required interfaces are down, or the submitted evidence is insufficient to verify interface status. 0 ptsNo MarksNo evidence is provided showing interface status, or the submitted evidence does not demonstrate that any required interfaces are operational. | 8 pts |
| This criterion is linked to a Learning OutcomeIP Addressing Configuration | 8 ptsFull MarksAll required routers and switches are configured with the correct IP addressing as specified. Submitted evidence (for example, output from the command "show ip int brief") clearly shows valid IP addresses on all required interfaces with no missing or incorrect configurations. 7 ptsMost MarksIP addressing is correctly configured on most required interfaces, but one interface has a minor addressing issue (such as a missing, incorrect, or misassigned IP address). Evidence shows correct addressing on the remaining interfaces. 5 ptsModerate MarksIP addressing is configured on some required interfaces, but multiple interfaces have missing or incorrect IP addresses. Evidence shows partial completion of the IP addressing requirement. 3 ptsSome MarksMinimal IP addressing configuration is present. One or more interfaces may have an IP address assigned, but most required interfaces are missing or incorrectly configured, or evidence is insufficient to verify addressing. 0 ptsNo MarksNo evidence is provided showing IP addressing configuration, or the evidence does not demonstrate that any required interfaces have been configured with IP addresses. | 8 pts |
| This criterion is linked to a Learning OutcomeSecure Device Access Configuration | 8 ptsFull MarksAll required devices are configured with secure access settings as specified. Submitted evidence (for example, output from the command "show run ¦ section line") clearly shows an enable password, console password, and telnet access configured on all required devices with no missing elements. 7 ptsMost MarksSecure access settings are correctly configured on most required devices, but one device is missing a required access element (such as an enable password, console password, or telnet configuration). Evidence otherwise shows secure access configured on the remaining devices. 5 ptsModerate MarksSome secure access settings are configured, but multiple required devices are missing one or more required access elements. Evidence shows partial completion of the secure device access requirement. 3 ptsSome MarksMinimal secure access configuration is present. One or more access elements may be configured on a limited number of devices, but most required devices are missing secure access settings, or evidence is insufficient to verify configuration. 0 ptsNo MarksNo evidence is provided showing secure device access configuration, or the submitted evidence does not demonstrate that any required devices have secure access settings configured. | 8 pts |
| This criterion is linked to a Learning OutcomeVLAN Creation and Trunk Configuration | 8 ptsFull MarksThe required VLAN is created on all SITE1 switches, access ports are assigned as specified, and trunk links are configured to carry the required VLANs. Submitted evidence (for example, output from "show vlan" and "show interfaces trunk") demonstrates correct VLAN and trunk configuration across all switches. 7 ptsMost MarksVLAN and trunk configuration is mostly correct, but one switch has a minor issue (such as a missing VLAN, incorrect access port assignment, or trunk configuration problem). Evidence shows correct configuration on the remaining switches. 5 ptsModerate MarksVLAN and trunk configuration is partially complete. The required VLAN exists on some switches or trunking is configured on some links, but multiple required switches or links are missing correct configuration. 3 ptsSome MarksMinimal VLAN or trunk configuration is present. One or more switches may show VLAN or trunk configuration, but most required switches or links are not correctly configured, or evidence is insufficient to verify setup. 0 ptsNo MarksNo evidence is provided showing VLAN creation or trunk configuration, or the submitted evidence does not demonstrate that any required VLAN or trunk settings are configured. | 8 pts |
| This criterion is linked to a Learning OutcomeInterVLAN Routing (R1-SITE1) | 8 ptsFull MarksInterVLAN routing is correctly configured on R1-SITE1, allowing traffic to route between the required VLANs. Submitted evidence (for example, output from "show run ¦ section interface") demonstrates correctly configured subinterfaces with no missing or incorrect elements. 7 ptsMost MarksInterVLAN routing is mostly correct, but one minor issue is present (such as a missing, incorrect, or misconfigured subinterface). Evidence otherwise shows routing configured between the VLANs. 5 ptsModerate MarksInterVLAN routing is partially configured. One or more required subinterfaces are present, but routing between VLANs is incomplete or incorrectly configured. 3 ptsSome MarksMinimal interVLAN routing configuration is present. Evidence shows an attempt to configure routing on R1-SITE1, but required VLAN routing is largely missing or incorrect. 0 ptsNo MarksNo evidence is provided showing interVLAN routing configuration on R1-SITE1, or the submitted evidence does not demonstrate any routing between VLANs. | 8 pts |
| This criterion is linked to a Learning OutcomeDHCP Configuration | 8 ptsFull MarksDHCP is correctly configured as specified, and submitted evidence (for example, output from "show run ¦ section ip dhcp") shows a complete DHCP configuration with required pools and exclusions present. 7 ptsMost MarksDHCP is mostly configured correctly, but one minor issue is present (such as a missing exclusion or incomplete pool configuration). Evidence otherwise shows DHCP functioning as intended. 5 ptsModerate MarksDHCP is partially configured. A DHCP pool exists, but required elements are missing or incorrect, and the configuration does not fully meet the specified requirements. 3 ptsSome MarksMinimal DHCP configuration is present. Evidence shows an attempt to configure DHCP, but required pools or settings are largely missing or incorrect. 0 ptsNo MarksNo evidence is provided showing DHCP configuration, or the submitted evidence does not demonstrate that DHCP is configured. | 8 pts |
| This criterion is linked to a Learning OutcomeNAT / PAT Configuration | 8 ptsFull MarksNAT / PAT is correctly configured as specified. Submitted evidence (for example, output from "show run ¦ section ip nat" or "show ip nat statistics") demonstrates source NAT with overload configured and operating on the required device. 7 ptsMost MarksNAT / PAT is mostly configured correctly, but one minor issue is present (such as a missing statement or incorrect inside/outside designation). Evidence otherwise shows NAT / PAT configured. 5 ptsModerate MarksNAT / PAT is partially configured. Some required NAT statements or interface designations are present, but the configuration does not fully meet the specified requirements. 3 ptsSome MarksMinimal NAT / PAT configuration is present. Evidence shows an attempt to configure NAT, but required statements or interface roles are largely missing or incorrect. 0 ptsNo MarksNo evidence is provided showing NAT / PAT configuration, or the submitted evidence does not demonstrate that NAT / PAT is configured. | 8 pts |
| This criterion is linked to a Learning OutcomeStatic Default Route Configuration | 9 ptsFull MarksA static default route is correctly configured as specified. Submitted evidence (for example, output from "show ip route") clearly shows a default route pointing to the required next hop with no errors. 8 ptsMost MarksA static default route is present, but one minor issue exists (such as an incorrect next hop or incomplete configuration). Evidence otherwise shows a default route configured. 6 ptsModerate MarksA static default route is partially configured. Evidence shows an attempt to configure a default route, but the configuration does not fully meet the specified requirements. 3 ptsSome MarksMinimal default route configuration is present. Evidence shows an incomplete or incorrect default route, or the submitted evidence is insufficient to verify proper configuration. 0 ptsNo MarksNo evidence is provided showing static default route configuration, or the submitted evidence does not demonstrate that a default route is configured. | 9 pts |
| This criterion is linked to a Learning OutcomeOSPF Configuration on R2-SITE2 | 9 ptsFull MarksOSPF is correctly configured on R2-SITE2 as specified. Submitted evidence (for example, output from "show ip ospf interface") clearly shows an active OSPF process with the required interfaces participating and no configuration issues present. 8 ptsMost MarksOSPF is mostly configured correctly on R2-SITE2, but one minor issue is present (such as an incorrect network statement or missing passive interface). Evidence otherwise shows OSPF enabled and operating. 6 ptsModerate MarksOSPF is partially configured on R2-SITE2. Evidence shows an OSPF process present, but required interfaces are missing, misconfigured, or not participating as specified. 3 ptsSome MarksMinimal OSPF configuration is present. Evidence shows an attempt to configure OSPF on R2-SITE2, but the configuration does not meet the required specifications. 0 ptsNo MarksNo evidence is provided showing OSPF configuration on R2-SITE2, or the submitted evidence does not demonstrate that OSPF is configured. | 9 pts |
| This criterion is linked to a Learning OutcomeRedundant Links and Channel Groups | 9 ptsFull MarksRedundant links and channel groups are correctly configured as specified. Submitted evidence (for example, a logical view or relevant configuration output) clearly shows the required redundant links grouped correctly with no missing or incorrect channel group configuration. 8 ptsMost MarksRedundant links and channel groups are mostly configured correctly, but one minor issue is present (such as a missing link in a channel group or an incorrectly configured group). Evidence otherwise shows redundant links grouped as intended. 6 ptsModerate MarksRedundant links and channel groups are partially configured. Evidence shows some links grouped into channel groups, but multiple required links or groups are missing or incorrectly configured. 3 ptsSome MarksMinimal redundant link or channel group configuration is present. Evidence shows an attempt to configure redundant links, but most required links are not correctly grouped, or evidence is insufficient to verify proper configuration. 0 ptsNo MarksNo evidence is provided showing redundant links or channel group configuration, or the submitted evidence does not demonstrate that any required redundant links are configured. | 9 pts |
| This criterion is linked to a Learning OutcomeSpanning Tree Configuration | 9 ptsFull MarksSpanning Tree is configured as specified. Submitted evidence (for example, output from "show spanning-tree") clearly shows the correct root bridge placement and expected spanning-tree state with no configuration issues present. 8 ptsMost MarksSpanning Tree is mostly configured correctly, but one minor issue is present (such as incorrect root priority on one device). Evidence otherwise shows spanning-tree enabled and operating. 6 ptsModerate MarksSpanning Tree is partially configured. Evidence shows spanning-tree enabled, but the root bridge or spanning-tree state does not fully match the specified requirements. 3 ptsSome MarksMinimal spanning-tree configuration is present. Evidence shows spanning-tree activity, but required root placement or configuration is largely missing or incorrect. 0 ptsNo MarksNo evidence is provided showing spanning-tree configuration, or the submitted evidence does not demonstrate that spanning-tree is configured. | 9 pts |

Total Points: 100

This is a significant milestone for your IT 350 course. Looking at the topology and the rubric, you have a classic "Enterprise Branch" scenario involving two sites, a provider core, and several advanced Layer 2 and Layer 3 technologies.

Since you are approaching this as a junior network engineer, we will tackle this systematically. We’ll build from the "ground up"—starting with basic connectivity and security, then moving to VLANs, routing, and finally optimization.

Phase 1: Basic Management & Security

Before you can manage a network, you must secure the "doors." You need to apply these steps to R1-SITE1, R2-SITE2, S1-SITE1, S2-SITE1, and S3-SITE1.

- Hostname Configuration:

  - Enter Global Configuration mode: configure terminal.

  - Set the name: hostname R1-SITE1 (Repeat for each device using the exact labels from the topology).

- Secure the Lines:

  - Console: line con 0 -> password [yourpassword] -> login.

  - VTY (Telnet/SSH): line vty 0 15 -> password [yourpassword] -> login.

  - Privileged Exec: exit (back to global config) -> enable password [yourpassword].

- Prevent DNS Lookup Hangs: (Optional but helpful for novices) Run no ip domain-lookup so the CLI doesn't freeze when you mistype a command.

Phase 2: Layer 2 Infrastructure (SITE1 Switches)

Your topology shows three switches at Site 1 (S1, S2, S3) connected in a triangle.

- VLAN Creation: On all three switches, create the required VLANs.

  - vlan [ID] -> name [Name].

- Trunking: Identify the links connecting the switches to each other and S1 to R1.

  - On those interfaces: switchport mode trunk.

- EtherChannel (Redundant Links): The dashed lines in your image indicate redundant links.

  - Select the range of ports (e.g., int range f0/1 - 2).

  - Group them: channel-group 1 mode desirable (or active).

- Spanning Tree (STP): To ensure S1 is the "boss" of the Site 1 network:

  - On S1-SITE1: spanning-tree vlan [ID] root primary.

Phase 3: Layer 3 - Inter-VLAN Routing & DHCP

Now we give the PCs a way to talk outside their own VLAN.

- Router-on-a-Stick (R1-SITE1):

  - Go to the interface connected to S1 (e.g., G0/1).

  - no ip address and no shut.

  - Create sub-interfaces: int g0/1.10 (for VLAN 10).

  - encapsulation dot1Q 10.

  - ip address [Gateway_IP] [Subnet_Mask].

- DHCP Server: Still on R1-SITE1, set up the pools so PCs get IPs automatically.

  - ip dhcp excluded-address [Start_IP] [End_IP] (Exclude your router and switch management IPs).

  - ip dhcp pool SITE1_POOL.

  - network [Network_ID] [Mask].

  - default-router [R1_Subinterface_IP].

  - dns-server 8.8.8.8.

Phase 4: WAN Connectivity & Routing

This connects Site 1 to Site 2 through the Provider.

- Static Default Route (R1-SITE1):

  - R1 needs to know where to send unknown traffic (the Internet).

  - ip route 0.0.0.0 0.0.0.0 [Provider_Router_IP].

- OSPF (R2-SITE2): Based on the rubric, Site 2 uses OSPF.

  - router ospf 1.

  - network [Site2_Network] [Wildcard_Mask] area 0.

  - passive-interface [Interface_to_PC3] (Security best practice).

Phase 5: NAT / PAT (Network Address Translation)

Usually configured on the edge router (R1 or R2) to allow private IPs to reach the "Public" Provider Router.

- Define Inside/Outside:

  - LAN Interface: ip nat inside.

  - WAN Interface: ip nat outside.

- Configure PAT:

  - Create an ACL for your LAN: access-list 1 permit [LAN_Network] [Wildcard].

  - Map it: ip nat inside source list 1 interface [WAN_Interface] overload.

Phase 6: Verification (The "Show" Commands)

Before you save, run these to ensure you hit the "Full Marks" criteria:

- show ip interface brief: Check for "up/up" (Green links).

- show vlan brief: Check that ports are in the right VLANs.

- show etherchannel summary: Ensure the group is "SU" (Switched/Up).

- show ip route: Verify the S* (Static) and O (OSPF) routes exist.

- Final Test: Can PC0 ping PC3?

Next Step:

To give you the exact IP addresses and VLAN IDs, please upload the .docx instructions. I have the topology from your screenshots, but the specific addressing scheme (IPs/Subnets) is in that Word document.

I don't necessarily need the PC2/PC3 screenshots yet, as the router configurations will reveal their intended settings! Want to start with Phase 1 and 2?

Based on the specific requirements in your lab document, here is the complete, step-by-step configuration for every piece of equipment.

1. R1-SITE1 (Gateway Router)

This router handles Inter-VLAN routing, DHCP, NAT, and the default route.

Bash

enable

configure terminal

hostname R1-SITE1

# Security Requirements

enable secret cr3dit

line con 0

password v@liant

login

line vty 0 4

password CTR123

login

exit

# WAN Interface - Subnet 216.127.251.0/30

# Provider uses .1, R1-SITE1 uses .2

interface GigabitEthernet0/0

ip address 216.127.251.2 255.255.255.252

ip nat outside

no shutdown

# Inter-VLAN Routing (Subinterfaces)

# VLAN 1 (Subnet 192.168.0.0/24)

interface GigabitEthernet0/1.1

encapsulation dot1Q 1

ip address 192.168.0.1 255.255.255.0

ip nat inside

# VLAN 20 (Subnet 192.168.128.0/25 for <100 hosts)

interface GigabitEthernet0/1.20

encapsulation dot1Q 20

ip address 192.168.128.1 255.255.255.128

ip nat inside

exit

interface GigabitEthernet0/1

no shutdown

# DHCP for VLAN 20

ip dhcp excluded-address 192.168.128.1 192.168.128.10

ip dhcp pool VLAN20_DHCP

network 192.168.128.0 255.255.255.128

default-router 192.168.128.1

dns-server 8.8.8.8

# NAT Overload (PAT)

access-list 1 permit 192.168.0.0 0.0.255.255

ip nat inside source list 1 interface GigabitEthernet0/0 overload

# Static Default Route to Provider

ip route 0.0.0.0 0.0.0.0 216.127.251.1

do wr mem

2. R2-SITE2 (Remote Site Router)

R2 is mostly configured but requires OSPF to connect to the Provider.

Bash

enable

configure terminal

hostname R2-SITE2

# Security Requirements

enable secret cr3dit

line con 0

password v@liant

login

line vty 0 4

password CTR123

login

exit

# OSPF Configuration (Area 0)

router ospf 1

network 216.127.251.8 0.0.0.3 area 0

network 192.168.1.0 0.0.0.255 area 0

passive-interface GigabitEthernet0/1

do wr mem

3. S1-SITE1 (Core Switch)

This switch is the Root Bridge for Spanning Tree and connects to the router.

Bash

enable

configure terminal

hostname S1-SITE1

# Security Requirements

enable secret cr3dit

line con 0

password v@liant

login

line vty 0 4

password CTR123

login

exit

# VLAN 20 Creation

vlan 20

name DEV

exit

# Management IP (> .200)

interface vlan 1

ip address 192.168.0.201 255.255.255.0

no shutdown

exit

ip default-gateway 192.168.0.1

# Trunking to Router

interface GigabitEthernet0/1

switchport mode trunk

# EtherChannel to S2 and S3

interface range FastEthernet0/23 - 24

channel-group 1 mode desirable

switchport mode trunk

# Spanning Tree Root

spanning-tree vlan 1 root primary

spanning-tree vlan 20 root primary

do wr mem

4. S2-SITE1 (Distribution Switch)

Part of the redundant triangle in Site 1.

Bash

enable

configure terminal

hostname S2-SITE1

# Security Requirements

enable secret cr3dit

line con 0

password v@liant

login

line vty 0 4

password CTR123

login

exit

# VLAN 20 Creation

vlan 20

name DEV

exit

# Management IP

interface vlan 1

ip address 192.168.0.202 255.255.255.0

no shutdown

exit

ip default-gateway 192.168.0.1

# EtherChannel to S1 (Group 1)

interface range FastEthernet0/23 - 24

channel-group 1 mode desirable

switchport mode trunk

# EtherChannel to S3 (Group 2)

interface range FastEthernet0/21 - 22

channel-group 2 mode desirable

switchport mode trunk

do wr mem

5. S3-SITE1 (Access Switch)

Connects PC0 to the new DEV VLAN.

Bash

enable

configure terminal

hostname S3-SITE1

# Security Requirements

enable secret cr3dit

line con 0

password v@liant

login

line vty 0 4

password CTR123

login

exit

# VLAN 20 Creation

vlan 20

name DEV

exit

# Management IP

interface vlan 1

ip address 192.168.0.203 255.255.255.0

no shutdown

exit

ip default-gateway 192.168.0.1

# Assign PC0 to VLAN 20

interface FastEthernet0/1

switchport mode access

switchport access vlan 20

# EtherChannel to S2 (Group 2)

interface range FastEthernet0/21 - 22

channel-group 2 mode desirable

switchport mode trunk

# EtherChannel to S1 (Group 3)

interface range FastEthernet0/23 - 24

channel-group 3 mode desirable

switchport mode trunk

do wr mem

Verification Checklist

- PC0 IP: Open PC0 > Desktop > IP Configuration. Select DHCP. It should get an IP in the 192.168.128.x range.

- Ping Test: From PC0, ping PC3 (192.168.1.x). If NAT and OSPF are correct, this will succeed.

- Show CDP Neighbor: On S1-SITE1, run show cdp neighbor to verify all hostnames are correct.

- Spanning Tree: On S1-SITE1, run show spanning-tree to confirm it says "This bridge is the root".