# IT 255 Project Retrospective (40 Points)

Complete the project retrospective after completing each project. Each project entry will be awarded 5 points for answering the questions or providing the requested information. Submit your retrospective in Week 7. Note: Project #1 and/or #11 may not be required in your course. If so, skip these questions.

## Project #1 – Build a Twisted-Pair Cable [Classroom and GEC only]

- What tools did you use to create your cable? A crimper for attaching the connectors, a wire stripper to remove the outer jacket, RJ-45 connectors, and a network cable tester to verify the connections.

- How did you test your cable? I used a network cable tester. It checks for continuity on all eight wires and ensures the wire mapping is correct (i.e., that the pins are in the correct order on both ends).

- What is a crossover cable? What changes do you make in creating one? A crossover cable is a special type of Ethernet cable used to connect two similar devices directly, such as two PCs or two switches. To create one, I terminate one end of the cable using the T568A wiring standard and the other end using the T568B wiring standard. T568A and T568B differ only in the order of the green and orange wire pairs—Pins 1, 2, 3, and 6 are where the green and orange pairs are swapped between the two standards. The blue and brown pairs (pins 4, 5, 7, 8) remain the same.

## Project #2: Basic Switch Configuration

- List at least 3 commands you learned from this project.

- enable

- configure terminal (Also conf t)

- hostname [name]

- enable secret [password]

- What is the difference between the switch password and the enable secret password? The enable secret password is the modern, recommended method. It is always stored in the configuration using a strong MD5 hash. The older enable password command stores the password as plaintext (or with weak encryption if enabled). If both are configured, the enable secret password takes precedence.

- What command do you use to prevent your switch password from being in clear test?  service password-encryption.

## Project #3: Configuring DHCP and DNS

- Why is a DHCP Server a necessity when dealing with a complex network? A DHCP (Dynamic Host Configuration Protocol) server is essential because it automates the assignment of IP addresses and other network configuration settings (like subnet mask, default gateway, and DNS servers). In a complex network, this eliminates the enormous administrative overhead and potential for human error associated with manually configuring every single device.

- Why should a local network incorporate a DNS Server in the design? A local DNS (Domain Name System) server should be incorporated to translate human-readable hostnames (e.g., "fileserver1") into their corresponding IP addresses within the local network. This makes resources easier to access and remember for users, and it centralizes name management.

- It is not possible to incorporate an IP Reservation when using a Cisco Router for DHCP. How do you configure DHCP on a Cisco Router yet allow Static IP configurations? To allow for static IP configurations, you must exclude those addresses from the range that the DHCP server is allowed to lease. This is done using the ip dhcp excluded-address [start_ip_address] [end_ip_address] command in global configuration mode. This creates a reserved pool of addresses that will not be automatically assigned.

## Project #4: Subnetting and Supernetting

- What strategy did you use to assign your IP addresses? I used Variable Length Subnet Masking (VLSM), combined with Sequential Address Allocation.

- What is an advantage of using a /30 for links between routers? Using a subnet mask of /30 is highly efficient for point-to-point links between two routers. A /30 network provides exactly two usable host IP addresses, one for each end of the link. This conserves the maximum number of IP addresses by not wasting them on a larger subnet.

- What subnet mask would you use if you needed to support 700 computers on a network? To support 700 computers, I need to find the next power of 2 that is large enough to provide at least 700 host addresses. Therefore, the subnet mask would be a /22 or 255.255.252.0.

## Project #5: Advanced Switch Configuration

- What is the primary purpose of a VLAN? The primary purpose of a VLAN (Virtual Local Area Network) is to segment a physical network into multiple, logically separate networks. This enhances security by isolating traffic, improves performance by limiting the broadcast domain, and simplifies network management.

- What command do you use to verify VLAN configuration on a switch? The show vlan brief (OR sh vlan br) command is used to verify the VLAN configuration, showing a list of all configured VLANs and which ports are assigned to each.

- You have three VLANs configured on 2 connected switches. What must you do on the connected interfaces so that VLAN traffic is passed between the switches?  The interfaces connecting the two switches must be configured as trunk ports. A trunk port is capable of carrying traffic for multiple VLANs simultaneously by tagging frames with their VLAN ID using a protocol like 802.1Q.

## Project #6: Router Configuration

- What is the purpose of a router? The purpose of a router is to connect different networks together and forward packets between them. It operates at Layer 3 (the Network Layer) of the OSI model, making decisions based on IP addresses to determine the best path for traffic to reach its destination.

- What command do you use to see the routing table? show ip route (OR sh ip route).

- You have an unconfigured router connected to 3 networks on the following interfaces:

| Interface | Network |
| --- | --- |
| gi0/1 | 198.3.24.253/30 |
| gi0/2 | 192.168.3.0/24 |
| gi0/3 | 172.16.8.0/16 |

Your organization uses OSPF routing and uses area0. What commands are necessary to configure the router to advertise the connected networks?

configure terminal

router ospf 1

network 198.3.24.252 0.0.0.3 area 0

network 192.168.3.0 0.0.0.255 area 0

network 172.16.0.0 0.0.255.255 area 0

end

## Project #7: Implement a Wireless Network using WPA2 Enterprise Authentication

- What is the difference between WPA2 PSK and WPA2 Enterprise? WPA2-PSK uses a shared password for all users, while WPA2-Enterprise requires individual authentication through a server, offering stronger security and user management.

- Why is centralized authentication very important in a network with multiple access points? Centralized authentication is crucial because it provides a single point of management for all user access. This means you can add, remove, or modify user credentials in one place, and the changes apply across all access points. It ensures consistent security policies and enables seamless roaming for users between different Access Points.

- What is an improved solution for a RADIUS server (not available in Packet Tracer?  An improved solution is a modern Network Access Control (NAC) platform, such as Cisco Identity Services Engine (ISE) or Aruba ClearPass. These systems function as advanced RADIUS servers but also provide more granular control, including device profiling, posture assessment, and dynamic policy enforcement.

## Project #8: Wireshark Lab – Getting Started

- Did you experience any problems installing Wireshark? No, the installation was straightforward. However, common issues can include permissions problems or conflicts with antivirus software. It's also critical to correctly install the Npcap packet capture driver that comes bundled with Wireshark.

- What are two different ways you can start a Wireshark packet capture?

- From the Welcome Screen: Double-click the name of the network interface (e.g., "Wi-Fi" or "Ethernet") you want to capture traffic from.

- Using the Menu: Go to the Capture menu, select Options..., choose the desired interface from the dialog box, and click the Start button.

- What is the difference between packets if you repeat the lab without clearing the browser cache?  When repeating the lab without clearing the cache, you will see significantly fewer packets, especially HTTP GET requests for website assets like images, CSS, and JavaScript files. This is because the browser serves these files from its local cache instead of requesting them from the web server again.

## Project #9: Wireshark Lab – Identify Hosts and Users

- What advantage is it to know the manufacturer of a device? Knowing the manufacturer (via the OUI portion of a MAC address) helps in identifying the type of device on the network. This is useful for inventory management, troubleshooting (e.g., knowing you're dealing with an HP printer vs. a Dell laptop), and security (e.g., identifying known vulnerabilities associated with a specific vendor's products).

- Use ipconfig /all to discover your MAC address. Use a MAC address lookup tool (https://macvendors.com/) and input your MAC address. Who is the vendor of your network card?  Intel Corporate.

- What is a .pcap file? Why would you want to save one? A .pcap (packet capture) file is the standard file format used by Wireshark and other traffic capture tools to save a record of captured network packets. I would save one to analyze traffic offline, share it with colleagues for collaborative troubleshooting, maintain a forensic record of a security incident, or perform long-term network performance analysis.

## Project #10: Configure Secure WAN Link and Network Redundancy

- Did adding the redundant link cause any issues with the network? No, it did not. However, adding a redundant link without proper configuration can cause significant issues like Layer 2 switching loops or Layer 3 routing instability. Protocols like Spanning Tree Protocol (STP) at Layer 2 or dynamic routing protocols at Layer 3 are necessary to manage redundant paths and prevent these problems.

- What is the alternative to PPP encapsulation? A common alternative to PPP (Point-to-Point Protocol) encapsulation, especially in an all-Cisco environment, is HDLC (High-Level Data Link Control).

- What other authentication method is there besides CHAP?  Besides CHAP (Challenge-Handshake Authentication Protocol), another common method is PAP (Password Authentication Protocol). However, PAP is considered insecure because it sends the username and password in clear text.

## Project #11: Network Fingerprinting

- Did you have any issues installing nmap or zenmap? No issues were encountered. A common installation hurdle could be security software (antivirus/firewall) flagging Nmap as a hacking tool or issues with the Npcap driver installation, similar to Wireshark.

- Did you find Zenmap easier to use than nmap? Why? Yes. The primary reason is that it provides a Graphical User Interface (GUI), which eliminates the need to memorize Nmap's command-line syntax. It also offers pre-configured scan profiles, graphical topology maps, and easier ways to save and compare scan results.

- What does this nmap command do: nmap -O -p80,443 192.168.1-30?  
This command instructs Nmap to perform the following actions:

- -O: Enables OS detection to try and determine the operating system of the target hosts.

- -p80,443: Restricts the port scan to only port 80 (HTTP) and port 443 (HTTPS).

- 92.168.1.1-30: Defines the target range as the IP addresses from 192.168.1.1 through 192.168.1.30.

## Project #12: Capstone Project

- How comfortable are you using Packet Tracer after taking this course? After completing the projects in this course, I feel very comfortable using Packet Tracer. I am confident in my ability to build complex network topologies, configure various devices like routers and switches, and simulate network traffic to test configurations.

- What problems did you have creating your infrastructure? Some initial challenges involved ensuring proper IP addressing and subnetting across the entire infrastructure. Another difficulty was troubleshooting routing protocol configurations, such as OSPF, to ensure full connectivity between all networks without routing loops.

- What do you wish you had studied more in depth prior to the Capstone Project?  I wish I had spent more time studying the finer details of WAN technologies and encapsulation protocols. A deeper understanding in these areas would have made the design and troubleshooting phases of the capstone project even smoother.